��������������������������������������������������������������������������Ŀ

�       Highlight ODI Driver Installation for Novell Advanced NetWare      �

����������������������������������������������������������������������������

1.0  Novell NetWare ODI Driver Installation

�������������������������������������������������

Follow the steps outlined below to create a driver diskette for use

with Novell Advanced NetWare.

1.1  Preparation

�������������������������������������������������

Before you start with the workstation installation process make sure

you have the following items on hand:

1) Highlight Ethernet adapter card, as follows :

* HE-100 (for Novell Ethernet NE1000 compatible)

* HE-200, HE-200C, HE-200CB, HE-200CT, HE-205N, HE-205NB,

HE-205NT (for Novell Ethernet NE2000 compatible)

* HE-205 (for Novell Ethernet NE2000 / WD8013 compatible)

* HE-206 (for Novell Ethernet NE2000 compatible)

2) Highlight Ethernet driver diskette

3) Novell WSGEN diskette.

Create a BOOT diskette (this diskette may be self-booting, or it may

simply be a DOS-formatted blank diskette).

1.2  How To Install...

�������������������������������������������������

1) Copy the following five files into your BOOT disk.

a. LSL.COM      �� supplied by Novell in the WSGEN diskette.

b. HE200.COM    �� HE200 ODI Driver supplied by Highlight in the

\ODI\DOSODI subdirectory of the Ethernet

adapter diskette.

Note: 1. The Highlight DOSODI driver is an auto-configuration with the

original hardware setting in ethernet card.

2 Driver description: Highlight HE200 Ethernet MLID  v1.45

c. IPXODI.COM   �� supplied by Novell in the WSGEN diskette.

d. NETx.COM     �� supplied by Novell in the WSGEN diskette.

� or �

EMSNETx.EXE  �� (NETx program with EMS support)

� or �

XMSNETx.EXE  �� (NETx program with XMS support)

� or �

VLM.EXE      ��  supplied by Novell (Virtual Loadable Module program)

e. NET.CFG      �� Optional configuration file supplied by

Highlight in the \ODI\DOSODI subdirectory

of the Driver diskette. The HE200 ODI

driver has an automatically setting hardware

configuration routine, so that you may not

set INT level in NET.CFG. If you install more

than one LAN adapter in station, the PORT option

MUST be set in NET.CFG file.

2) After booting the workstation, type the following commands on

the console:

LSL      <Enter>

HE200    <Enter>

IPXODI   <Enter>

NETx     <Enter>

You may wish to write all commands into a batch file and execute the

batch file instead of typing all the commands each time the workstation

needs to be booted up.

1.3  NET.CFG for HE200

����������������������

The NET.CFG file is used to provide the software configuration to the

ODI driver on the NetWare workstation.  Typically, all of the required

operating parameters will have been preset by HE200 jumpers and matched

with file setting.

You can edit the NET.CFG file that is on the current directory in

order to change, add or delete any desired options by way of a plain-

ASCII text editor (DOS 5.0's EDIT and EDLIN commands are good examples

of plain-ASCII text editors).

�������������������������������������������������������������Ŀ

� IMPORTANT: There can only be ONE file named NET.CFG in the  �

� same floppy disk or pathed directory.  If the driver finds  �

� more than one NET.CFG file, it will become confused and may �

� not work reliably.  MAKE SURE THERE IS ONLY *ONE* NET.CFG!  �

� If there are two more NET.CFG files existing in different   �

� directories, then make sure that the one you choose must be �

� in the same directory with LSL.COM.                         �

���������������������������������������������������������������

PORT   : Indicates the base I/O address for the HE200 network card.

No need setting, when use only one LAN adapter

type  : [hex]

range : [0x300,0x320,0x340,0x360,0x240,0x280,0x2C0] for HE200 card,

MEM    : This option specifies a memory range to be used by the

network board. only support the WD mode ethernet board.

IRQ CHECK OFF : This parameter setting indicates the interrupt conflit

check routine is disable.

This parameter option only effect on DOS ODI driver for

MLID v1.43 or above. It can prevent using same interrupt

level from other devices. The DEFAULT setting is ENABLE

and ignore in NetWare's diskless environment.

1.3.1 Example NET.CFG

�������������������������������������������������

Following is an example fragment from a NET.CFG file.  This fragment

lists the HE200 options described above.  If this is the first time you

will be creating a NET.CFG file, note the layout of the NET.CFG fragment

below.  Spacing in the layout of the NET.CFG is important for reliable

operation of the adapter and network connection.

��> Link Driver HE200

�����> frame Ethernet_802.3

�����> protocol IPX 0 ETHERNET_802.3

�

��� The line "Link Driver HE200" must not be indented at all.

2.0  Novell NetWare ODI Server Driver Installation

��������������������������������������������������

Installing the Ethernet drivers into a server computer is outlined below.

2.1  Preparation

��������������������������������������������������

Before you start with the driver installation process to the server,

make sure you have the following on hand:

1) HE200 Ethernet adapter card

2) HE200 Driver diskette

2.2  How To Install...

��������������������������������������������������

Installation of the Ethernet driver to a server is broken down into two

parts: Hardware Installation (changes to existing setting in the adapter)

and Software Installation (copying the appropriate driver to the server

and loading).

2.2.1 Hardware Install

��������������������������������������������������

Installing the HE200 into a server is the same with the installation of

workstation. You just setup the hardware configuration by jumpers, and

record the final setting values.

2.2.2 Software Install

��������������������������������������������������

1) Login to the server as the "Supervisor" user.

2) Copy the HE200 driver supplied in the Software diskette's \ODI\386ODI

\V31(for NetWare v3.1x) subdirectory or \ODI\386ODI\V40(for NetWare

v4.0 or v3.12) subdirectory into the server's SYS:SYSTEM directory.

Also copy the HE200 driver supplied in the Software diskette's

A:>\ODI\386ODI\V40 to A:\ for install LAN Driver Options.

3) Use the LOAD command from the System Console to load the HE200.LAN

file into the server's memory.  Normally, the driver would load from

the SYS:SYSTEM directory of NetWare partition, but it could optionally

be loaded from a floppy disk or a directory on a DOS partition of the

server's hard disk instead.

A typical command line to load the server driver is:

LOAD HE200.LAN PORT=300, INT=3

The LOAD line options may be entered in any sequence and the commas

in between them are optional, as are most of the options themselves.

However, the PORT and INT options MUST be included and they MUST equal

the physical jumper settings.

Following are the parameters that may be included in the LOAD line:

PORT : The Base I/O Address at which the HE200 hardware has been set.

INT  : This is the interrupt level (IRQ) that the adapter has been

configured to use.

4) You may optionally wish to put the LOAD line in the STARTUP.NCF or

AUTOEXEC.NCF auto-load driver files so that the LOAD statement will

take effect when the server system is initialized.

3.0  Hardware Error Message

3.1  Error Message for HE200.COM

��������������������������������������������������

HE200-DOS-201: Ethernet Base I/O not found !

HE200-DOS-202: Ethernet IRQ error !

HE200-DOS-203: Ethernet Memory failure !

HE200-DOS-204: Ethernet Physical Address setting failure !

HE200-DOS-205: LAN adapter is not compatible with Highlight LAN adapter !

HE200-DOS-206: Ethernet Base I/O conflict occured.

HE200-DOS-207: IRQ has already used by other device !

HE200-DOS-210: Offline, check cabling system !

HE200-DOS-211: Carrier sense lost, check the terminator !

HE200-DOS-213: Transmit collided, check cabling system !

Closing Notes

��������������������������������������������������

the Novell NetWare "ODI Shell for DOS" provides more information

about the usage of NET.CFG files.

=================== FILE ODI\INSTALL.DOC -- END OF FILE ====================