����������������������ͻ

� Introduce SETETH.COM �

����������������������ͼ

SETETH.COM is a diagnostic program for ethernet lan board, it support

ethernet board like NE2000, NE1000, WD8003, WD8013, Highlight ethernet board

(i.e. HE-100, HE-200, HE-200B, HE-200C, HE-205, HE-206 etc.).

The information of valid SETETH.COM and HE-206 NIC setup program as follow,

SETETH   COM     13602 09-01-95   5:38p

SETETH   DOC     22918 09-01-95   6:05p

SETUTP   BAT        41 03-15-94   3:54p

SETBNC   BAT        41 03-15-94   3:54p

W906DM   EXE    217148 11-25-94   6:41p

W906DM   HLP     15373 12-23-94   1:57p

W906DM   DAT       110 12-23-94   1:57p

MAXISIZE PKT      1500 12-23-94   1:57p

MINISIZE PKT        50 12-23-94   1:57p

RUNTSIZE PKT        13 12-23-94   1:57p

[Features]

. Hardware configuration settings searching

(i.e. interrupt number, base memory address, port, node address. etc.)

. Netware transmission diagnosis

. Command line software setting for jumperless ethernet board, syntax as

follows,

=====   Usage: SETETH [options]

Options summary

-h = help screen (or -?)

-p:<parameters> = setting the AT/LANTIC(DP83905 or W89C906) in runtime

configuration or writing this configuration into the EEPROM

Parameters summary

IOnnn = setting controller's I/O base address

nnn: 300, 240, 280, 2C0, 320, 340, 360

IRQnn = setting LAN board's Interrupt level

default=> nn: 2, 3, 4, 5, 9, 10, 11 (in direct mode)

nn: 2, 3, 4, 5, 9, 10, 11, 12, 15 (in coded mode)

<COD|DIR> = setting interrupt mode

COD = ethernet board(HE-205) for 8 interrupts

DIR = ethernet board(HE-206) for 6 interrupts

<NE|WD> = setting LAN board's memory buffer access mode

NE = compatible with Novell's NE2000 mode

WD = compatible with the Ethercard PLUS16 mode

<TPI|BNC|AUI> = setting physical layer interface

TPI = 10BASE-T Compatible SequelchLevel

BNC = Thin Ethernet(10BASE2)

AUI = Thick Ethernet(10BASE5)

FRn = setting Fast Read, this function only effect on NE2000 mode

n: 0 = normal fetch data

1 = fetch data before the current IORD has completed (fast).

=====   Press any key to continue .....

<AT|NAT> = setting I/O CHRDY for timing compatibility, this function

only effect on NE2000 mode

AT  = ISA machines for IBM PC/AT compatible, *** default ***

NAT = COMPAQ 286 Protoble machines, IBM PC Model 30-286

machines, Machines that use the Chips and Technologies

chip set

<IO16CN|IO16CY> = setting LAN board's /IO16 control signal line, this

function only effect on NE2000 mode

IO16CN = generated only on address decode, *** default ***

IO16CY = generated after/IORD or /IOWR on address decode

ROMnn = setting Boot ROM base memory address

nn: 00             No Boot ROM, *** default ***

C0     0C000H     8k/16k

C4     0C400H     8k/16k

C8     0C800H     8k/16k

CC     0CC00H     8k/16k

D0     0D000H     8k/16k

D4     0D400H     8k/16k

D8     0D800H     8k/16k

DC     0DC00H     8k/16k

C03    0C000H    32k  not available

C83    0C800H    32k  not available

D03    0D000H    32k  not available

D83    0D800H    32k  not available

C06    0C000H    64k  not available

D06    0D000H    64k  not available

<SR|WE> = programming option

SR = setting the configuration into the AT/LANTIC(DP83905 or

W89C906) configuration registers, *** default ***

WE = Writing the AT/LANTIC(DP83905 or W89C906) configuration

into EEPROM

-s = Show the AT/LANTIC(DP83905 or W89C906) runtime configuration

[Note]:1. All options must be direct after the "-" or "/".

2. The priority order : "H" > "P" > "S", and these options

do not execute network diagnosis.

The ethernet board for non-software configuration will ignore

the options("P", "S" etc.).

3. The option "H" will ignore the other's options.

4. The parameters "COD" or "DIR" must be setting ahead of "IRQnn".

5. As Unknow IRQ mode condition, it must run SETETH /P:IRQnn to correct.

=====   End of software setting description

[Future Expand Features - Extra Menu Diagnostic function]

The follow features are not available now.

.View current hardware configuration

.View current hardware configuration registers' setting

.Change hardware configuration registers' setting

.Network node communication check

.Diagnosis

.Write hardware configuration registers' setting to EEPROM

[End of Features]

���������������������ͻ

�  Using  SETETH.COM  �

���������������������ͼ

The screen message appears when you use the "SETETH.COM" under DOS prompt.

The follows show the screen message formats and explanations.

���� Title of SETETH ������������������������������������������������������Ŀ

�          _____ The HCS_Network_utility, Version 3.0 _____                 �

�SetEth quick identify the Ethernet hardware configuration and Network      �

�(C) Copyright Highlight computer systems, Inc.   1991-1995                 �

�Authorized by EDWIN WANG.                                                  �

�����������������������������������������������������������������������������

<Explanation> :

This is a title message appears when you use the "SETETH.COM" program each

time.

���� Diagnosys of Ethernet ������������������������������������������������Ŀ

�Lan_Board type : ______ Ethernet (__KB, __ bit)                            �

�Lan Chip : ________ chip !                                                 �

�Hardware Configuration : IRQ = _, I/O Base = ___h, NodeID = 0x____________ �

�                                                                           �

�Ethernet simple diagnosis ..... _____________                              �

�����������������������������������������������������������������������������

<Explanation> :

[Lan_Board type : ______ Ethernet (__KB, __ bit)]

This item show the ethernet's hardware mode, RAM buffer size and

bus width. There are NE1000, NE2000, WD8003, WD8013 four types of

ethernet hardware mode can be found in this field;   and the RAM

buffer size will be from 6KB upto 16KB range, it is a memory error

when RAM buffer size is below 6KB;  the bus width field can be 8

or 16.

[Lan Chip : ________ chip !]

The follows chips may be found,

1. WB89C90, NS83C90, Unknow brand NIC90 chip.

2. WB89C906, UM9003, Unknow brand NIC904.

3. WB89C905, MX98905, NS83C905, Unknow brand NIC905.

[IRQ = _]

This item show the ethernet interrupt level in decimal number.

If the IRQ = "?" mark, it means that the interrupt circuit has

not activity, and the screen would display "Ethernrt IRQ error !"

message in previous time. The follows are the "Ethernrt IRQ error"

situation,

1. IRQ jumper is not setting.

2. IRQ wire is cut off or short with other wire.

3. The IRQ setting may be conflict with other interface.

4. Network controller is damage.

<Note> : IRQ = interrupt level, wrong IRQ level setting may cause

reveiving error in network.

[I/O Base = ___h]

This item show the ethernet Base I/O port address in hex-decimal

number. The I/O base address is in 32 sequence address range.

[NodeID = 0x____________]

This item show the ethernet's physical node address in hex-decimal

number.

[Ethernet simple diagnosis ..... _____________]

After completion of hardware configuration, it will execute the

network diagnosis.         See Appendix.

���� Information of configuration registers �������������������������������Ŀ

�       NIC configuration registers information                             �

�                                                                           �

�Lan Board Chip = ________ chip !                                           �

�Hardware Data Bus Access mode = _____________________                      �

�Physical interface = _________________________________                     �

�I/O Address = ____________________                                         �

�IRQ level = ________________________________                               �

�Boot ROM segment = ________________________                                �

�/IO16 control by /IORD or /IOWR = ___________________                      �

�I/O CHRDY timing compatibility = _________________________________         �

�Data read = _________________________                                      �

�����������������������������������������������������������������������������

<Explanation> :

If your lan board is a software setting ethernet board, then you could

use the "/S" option to show the information of runtime configuration

registers in the NIC chip (i.e. SETETH /S).  The follows are the option

of information message for each item,

[Lan Board Chip = ________ chip !]:

1. WB89C9064, UM9003, Unknow brand NIC904.

2. WB89C905, MX98905, NS83C905, Unknow brand NIC905.

[Hardware Data Bus Access mode = _____________________]:

1. I/O mode, set with NE

2. Shared RAM mode, set with WD

[Physical interface = _________________________________]:

1. 10BASE-T, set with TPI, link Good

2. 10BASE-T, set with TPI, link Bad

3. 10BASE2, set with BNC

4. 10BASE5, set with AUI

[I/O Address = ____________________]:

1. 300h, set with IO300

2. 240h, set with IO240

3. 280h, set with IO280

4. 2C0h, set with IO2C0

5. 320h, set with IO320

6. 340h, set with IO340

7. 360h, set with IO360

[IRQ level = ________________________________]:

* In Coded interrupt mode :

1. 3, set with IRQ3, in Coded mode

2. 4, set with IRQ4, in Coded mode

3. 5, set with IRQ5, in Coded mode

4. 9, set with IRQ9, in Coded mode

or

2, set with IRQ2, in Coded mode

5. 10, set with IRQ10, in Coded mode

6. 11, set with IRQ11, in Coded mode

7. 12, set with IRQ12, in Coded mode

8. 15, set with IRQ15, in Coded mode

* InDirect interrupt mode :

1. 3, set with IRQ3, in Direct mode

2. 4, set with IRQ4, in Direct mode

3. 5, set with IRQ5, in Direct mode

4. 9, set with IRQ9, in Direct mode

or

2, set with IRQ2, in Direct mode

[Note]: The "Unknown original IRQ mode" means that the IRQ mode setting is

Differ from the ethernet board IRQ circuit.

[Boot ROM segment = ________________________]:

1. No boot ROM segment

2. C000h 16KB,set with ROMC0

3. C400h 16KB,set with ROMC4

4. C800h 16KB,set with ROMC8

5. CC00h 16KB,set with ROMCC

6. D000h 16KB,set with ROMD0

7. D400h 16KB,set with ROMD4

8. D800h 16KB,set with ROMD8

9. DC00h 16KB,set with ROMDC

10. C000h 32KB,set with ROMC03

11. C800h 32KB,set with ROMC83

12. D000h 32KB,set with ROMD03

11. D800h 32KB,set with ROMD83

12. C000h 64KB,set with ROMC06

13. D000h 64KB,set with ROMD06

14. Unknown boot ROM segment

=====  The below 3 items are only effect on NE2000 mode

[/IO16 control by /IORD or /IOWR = ___________________]:

1. No, set with IO16CN

2. Yes, set with IO16CY

[I/O CHRDY timing compatibility = _________________________________]:

1. Not IBM PC/AT compatible, set with NAT

2. IBM PC/AT compatible, set with AT

[Data read = _________________________]:

1. Normal read, set with FR0

2. Fast read, set with FR1

���� Information of software setting ��������������������������������������Ŀ

�The follows are the setting options :                                      �

�       NIC configuration registers information                             �

�                                                                           �

�Lan Board Chip = ________ chip !                                           �

�Hardware Data Bus Access mode = _____________________                      �

�Physical interface = _________________________________                     �

�I/O Address = ____________________                                         �

�IRQ level = ________________________________                               �

�Boot ROM segment = ________________________                                �

�/IO16 control by /IORD or /IOWR = ___________________                      �

�I/O CHRDY timing compatibility = _________________________________         �

�Data read = _________________________                                      �

�                                                                           �

�Write current configuration to NIC configuration registers                 �

�Are you sure? (Yes/No)                                                     �

�This configuration chang is only for temporary use, after power off        �

�it will be lose !                                                          �

�The runtime configuration setting about Boot ROM address and Interrupt mode�

�did not change !                                                           �

�Change successfully                                                        �

�����������������������������������������������������������������������������

<Explanation> :

If your lan board is a software setting ethernet board, then you could

use the "/P:" option to setup the runtime configuration registers in

NIC chip or write configurations to the lan board's EEPROM.

The option setting information message for each item will display on the

screen after you using setup function (i.e. SETETH /P:io300 irq3 nat sr).

Please refer to the explanation of the Information of configuration

registers in previous section.

When using "WE" parameter then will notify message on screen as

"Write current configuration to EEPROM".

When using "SR" parameter then will notify message on screen as

"Write current configuration to NIC configuration registers".

When using "WE SR" parameter then will notify message on screen as

"Write current configuration to both NIC configuration registers

and EEPROM".

[Note]: The "Differ from original IRQ mode" means that the IRQ mode

setting is Differ from the ethernet board IRQ circuit.

����������������������ͻ

� Samples of Execution �

����������������������ͼ

-------------------------------------------------------------------

[example 1.] SETETH <enter>

���������������������������������������������������������������������������Ŀ

�          _____ The HCS_Network_utility, Version 3.0 _____                 �

�SetEth quick identify the Ethernet hardware configuration and Network      �

�(C) Copyright Highlight computer systems, Inc.   1991-1995                 �

�Authorized by EDWIN WANG.                                                  �

�                                                                           �

�Lan_Board type : NE2000 Ethernet (16KB, 16 bit)                            �

�Lan Chip : WB89C905 chip !                                                 �

�Hardware Configuration : IRQ = 3, I/O Base = 300h, NodeID = 0x484C0001032A �

�                                                                           �

�Ethernet simple diagnosis ..... Pass !                                     �

�����������������������������������������������������������������������������

or

���������������������������������������������������������������������������Ŀ

�          _____ The HCS_Network_utility, Version 3.0 _____                 �

�SetEth quick identify the Ethernet hardware configuration and Network      �

�(C) Copyright Highlight computer systems, Inc.   1991-1995                 �

�Authorized by EDWIN WANG.                                                  �

�                                                                           �

�Unknown original IRQ mode                                                  �

�Lan_Board type : NE2000 Ethernet (16KB, 16 bit)                            �

�Lan Chip : NS83C905 chip !                                                 �

�Hardware Configuration : IRQ = 5, I/O Base = 300h, NodeID = 0x484C00010019 �

�                                                                           �

�Ethernet simple diagnosis ..... 10BASE-T Good                              �

�����������������������������������������������������������������������������

-------------------------------------------------------------------

[example 2.] SETETH -P:IO320 IRQ4 NE TPI FR0 AT ROMC8 WE <enter>

���������������������������������������������������������������������������Ŀ

�          _____ The HCS_Network_utility, Version 3.0 _____                 �

�SetEth quick identify the Ethernet hardware configuration and Network      �

�(C) Copyright Highlight computer systems, Inc.   1991-1995                 �

�Authorized by EDWIN WANG.                                                  �

�                                                                           �

�The follows are the setting options :                                      �

�       NIC configuration registers information                             �

�                                                                           �

�Lan Board Chip = WB89C905 chip !                                           �

�Hardware Data Bus Access mode = I/O mode, set with NE                      �

�Physical interface = 10BASE-T, set with TPI                                �

�I/O Address = 320, set with IO320                                          �

�IRQ level = 4, set with IRQ4, in Coded mode                                �

�Boot ROM segment = C800 16KB, set with ROMC8                               �

�/IO16 control by /IORD or /IOWR = No, set with IO16CN                      �

�I/O CHRDY timing compatibility = IBM PC/AT compatible, set with AT         �

�Data read = Normal read, set with FR0                                      �

�                                                                           �

�Write current configuration to EEPROM                                      �

�����������������������������������������������������������������������������

Appendix  A   NETWORK DIAGNOSIS

[Network diagnosis]

After completion of hardware configuration, it will execute the

network diagnosis. The following is the executive message,

������ Execute message ���������������������������������������������������Ŀ

�Ethernet simple diagnosis ..... Pass !					   �

�			or						   �

�Ethernet simple diagnosis ..... Transmit collided on the network !	   �

�			or						   �

�Ethernet simple diagnosis ..... Carrier sense lost, check the terminator !�

�			or						   �

�Ethernet simple diagnosis ..... Offline, check cabling system !	   �

�			or						   �

�Ethernet simple diagnosis ..... 10BASE-T Good				   �

�			or						   �

�Ethernet simple diagnosis ..... 10BASE-T Bad				   �

����������������������������������������������������������������������������

<Explanation> :

[pass !] :

The transportation of packet has worked well.

[Transmit collided, check cabling system ! ] :

. Check the terminator.

. Number of packets are transmitting in same time so that

transmission is difficult or impossible.

It happen as follow condition,

1. Too mush network traffic.

2. The cable is not connect properly.

3. Too mush noise in cable system.

4. Wrong cable type.

[Carrier sense lost, check the terminator !] :

May be cable's length is too long.

It happen as follow condition,

1. One end of cable is not connect to terminator. (only BNC)

2. Terminator is not a 50 ohm resistor. (only BNC)

3. Too long of cable length.

4. Wrong cable type.

[Offline, check cabling system !] :

The interface board has be disconnected from ethernet cable.

It happen as follow condition, (only BNC mode)

1. The cable is not connect to BNC.

2. The cable's wire is cut off.

3. The ethernet transceiver is damage.

[10BASE-T Good or 10BASE-T Bad] :

This message is for jumperless ethernet board in UTP interface.

Appendix  B   Other Error Message

������ Error message ����������������������������������������������������Ŀ

� The Lan_board Base I/O could not be found, force to set NIC905 default. �

� Ethernet Base I/O not found !                                           �

���������������������������������������������������������������������������

<Explanation> :

1. If the Base I/O setting is not match(200h, 220h, 240h,

260h, 280h, 2A0h, 2C0h, 2E0h, 300h, 320h, 340h, 360h,

380h, 3A0h, 3C0h, 3E0h.)

2. If I/O base conflict with other interface I/O address.

3. ISA BUS on motherboard is damage. >> Change to next slot.

4. May be ETHERNIC controller failure.

������ Error message ���Ŀ

� Ethernet IRQ error !   �

��������������������������

<Explanation> :

1. IRQ jumper is not setting.

2. IRQ wire is cut off or short with other wire.

3. The IRQ setting may be conflict with other interface.

4. Network controller is damage.

<Note> : IRQ = interrupt level, wrong IRQ level setting may cause

reveiving error in network.

������ Error message ��������Ŀ

� Ethernet Memory error !     �

�������������������������������

<Explanation> :

1. Check your interface board's AT type switch (or jumper),

was it setting correct AT type.

2. May be wrong with interface board's memory buffer circuit.

3. ISA address BUS on motherboard is damage.

4. Memory electronic circuit is damage.

5. It must be at least 6 K Bytes memory range OK.