Driver Installation Guide for MS Windows 95

=================================================

Introduction

============

This document describes the procedures to install MS Windows 95 driver

for NE2000 Compatible Ethernet Adapter.

Installation procedure

======================

1. When log into Windows 95, click on the "Start" icon ,then move the

mouse to "Settings" icon and click on the "Control Panel".

2. In the Control Panel window, click on the "Network" icon.

3. In the Network Configuration dialog box, choose the "Add.." button.

4. When the Select Network Component Type dialog box appears, select

"Adapter" icon and then press "Add" button.

5. In the Selected Network adapters dialog box, Choose the "Have disk"

button.

6. Insert the driver diskette into drive A or B.

7. When the Install From Disk dialog box appear, specify the driver path

(A:\WIN95 or B:\WIN95) and then choose "OK" button.

8. Then, you will see "NE2000 Compatible Ethernet Adapter" list in the

dialog box, choose "OK" button.

9. The Windows 95 OS begin to copy driver from diskette and some system

files from Windows 95 installation CD or diskette.

10. Then, you will see "NE2000 Compatible Ethernet Adapter" list in the

Network Configuration dialog box, select it and then click on the

"Properties" button.

11. In this dialog box, select "Resources ",the screen will come up with

settings: IRQ  and  I/O address.  Make sure the  values that come up

match the actual board settings.

12. Restarting your system you will acquire network service.