Driver Installation Guide for Lantastic/AI

================================================

Introduction

============

This document describes the procedure to setup driver on LANtastic/AI

operating system for NE2000 Compatible Ethernet Adapter.

Installation Procedures

=======================

A. To install Lantastic/AI v4.0 or v5.0 software on a station:

1. Run your existing DOS system as you usually do.

2. Place the Lantastic/AI Network Operating System Disk #1 in

floppy drive A: or B:

3. Change to the drive containing the diskette and type

"INSATLL <Enter>" at the system prompt.

4. Choose NE2000 as your Ethernet Hardware Adapter.

5. Follow all the operations as you are prompted on the screen.

6. After installing the Netware Operating System, reboot your system.

--- Sample AUTOEXEC.BAT file ---

:

call C:\LANTASTI\STARTNET.BAT

:

--- Sample STARTNET.BAT file for Lantastic/AI v4.0 ---

:

@ECHO OFF

PATH C:\LANTASTI;%PATH%

SHARE /L:200

NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE

AILANBIO

REDIR ADMIN LOGINS=3

SERVER C:\LANTASTI.NET

NET LOGIN/WAIT \\ADMIN ADMIN

NET LPT TIMEOUT 10

:

--- Sample STARTNET.BAT file for Lantastic/AI v5.0 ---

:

@ECHO OFF

SET LAN_DIR=C:\LANTASTI.NET

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

SHARE /L:200

NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE

AILANBIO

REDIR ADMIN LOGINS=3

SERVER C:\LANTASTI.NET

NET LOGIN/WAIT \\ADMIN ADMIN

NET LPT TIMEOUT 10

NET POSTBOX

:

B. To install Artisoft LANtastic v6.0 LAN software on a workstation:

You can install LANtastic on computers that are running DOS only, or on

computers that are using both DOS and Windows. LANtastic v6.0 includes

both DOS- and Windows-cased interfaces.

If you run the command "INSTALL" under Windows, both the DOS and Windows

interface will be installed on the computer. If Windows isn't running,

only the DOS interface will be installed.

1. Run your existing DOS (or Windows) system as you usually do.

2. Place the first LANtastic diskette in floppy drive A: or B:

3. Run the LANtastic INSTALL programe.

a. If you are installing from the DOS prompt, type:

A: INSTALL <Enter> or B: INSTALL <Enter>

b. If you are starting from Windows. Form Windows Program Manager

or the File Manager windows, select the File menu and choose

RUN. In the RUN window, type:

A: INSTALL <Enter> or B: INSTALL <Enter>

then choose OK.

4. Proceed throught the installation screens, following the instruction

on each. Choose OK or press Enter to continue from each screen.

5. In the Select Network Adapter window select NE2000 adapter as your

Ethernet Hardware Adapter.

6. Follow all the operations as you are prompted on the screen.

7. When the installation is complete, INSATLL automatic reboot your

computer.