Driver Installation Guide for Windows For Workgroups 3.11

===============================================================

Introduction

============

This document describes the procedure to install Microsoft Windows

for Workgroups 3.11 driver for NE2000 Compatible Ethernet Adapter.

Installation Procedures

=======================

1. Turn your computer off, then Install the Adapter to your computer.

2. After the adapter installation, turn your computer on.

3. Start your Windows for Workgroups 3.11.

4. After Windows Start OK, Select "Network" icon and then Select

"Network  Setup" icon.

5. Click on the "Drivers..." button.

6. If you are replacing a previous LAN adapter select the Remove

button to delete that adapter

7. Click on the "Add Adapter" button.

8. Select "Unlisted or Updated Network Adapter" item bar on the Adapter

list.

9. Insert the driver diskette into drive A: or B: and enter A:\WFW311

or B:\WFW311

10. "NE2000 Compatible Ethernet Adapter" appears in the list and select

it, then Select "OK" bar.

11. "NE2000 Compatible Ethernet Adapter [NDIS3/NDIS2]" bar appears. It

indicates the NDIS driver has been copied into your Windows system.

12. Select "Close" bar and then Select "OK" bar.

13. Windows prompts you to insert several WFW system diskettes.

14. When message "Windows Setup has modified your SYSTEM.INI and

PROTOCOL.INI ........" appears, select "OK" bar.

15. Select "Restart Computer" bar.

16. Now, the Windows for Workgroup 3.11 is ready for your Ethernet

Adapter.