Driver Installation Guide for MS LAN Manager v2.x (DOS)

=============================================================

Introduction

============

This document describes the procedure to setup LAN Manager 2.x DOS

Workstation driver for NE2000 Compatible Ethernet Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedures

=======================

A. Suppose you install MS LAN Manager on a DOS workstation for the

first time.

1. Execute the "SETUP.EXE" program that is provided by LAN Manager.

Upon completion of retrieval of the last workstation disk's

contents, you will be asked if you want to import additional

network drivers.  Answer "YES".

2. Insert the driver diskette into the floppy drive.

3. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the driver disk and put them

on the harddisk and directory where LAN Manager resides.

4. After completion of the setup procedure, your CONFIG.SYS file will

contain statements for the NDIS driver program as sample file.

5. Reboot your computer and follow NDIS binding procedure to use the

NDIS driver programs.

B. If you have installed LAN Manager in your harddisk.

1. Execute the "SETUP.EXE" program.

2. On the screen, select 'Configuration', 'Network Drivers',

'other Drivers' function and input correct diskette driver name.

3. Insert the driver diskette into the floppy drive.

4. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the driver disk and put them

on the harddisk and directory where LAN Manager resides.

5. After completion of the setup procedure, your CONFIG.SYS file

will contain statements for the NDIS driver program.

6. Reboot your computer and follow NDIS binding procedure to use

the NDIS driver programs.

--- Sample CONFIG.SYS file ---

:

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHERNET\AN16\AN16.DOS

:

--- Sample PROTOCOL.INI file ---

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 6

NCBS = 12

BINDINGS = "AN16_NIF"

LANABASE = 0

[AN16_NIF]

DRIVERNAME = AN16$