***********************************************

*    Installation for Windows NT 3.5/3.51     *

***********************************************

1. Insert the driver floppy disk into drive A: or B:.

2. Double click the "Control Panel" in the "Main" Window after Windows NT

has been started.

3. Choose the "Network" icon in the "Control Panel" window.

4. In "Network Settings", click on "Add Adapter".

5. When adding a new adapter, the user should highlight the "<Other>

Require Disk from Manufacturer" item.

6. Input the drive A: or B: with path, \WINNT\35.  The network driver will

then be copied to the hard disk.

7. After users see the "CNet CN935 PCI Bus Ethernet Adapter" message, click

the OK button in the "Select OEM Option" window.

8. In the CN935 PCI Bus Adapter Setup window, the "Connection Type"

selection will appear on screen.  There are AutoSense, Twisted-Pair

(10Base T), Twisted-Pair Full_Duplex, Twisted-Pair No_Link_Test,

and BNC (10Base2).  User could click the "Configure..." button to

change the setting.

9. Return to the Network Settings window and click the binding button.

Then, click OK.

10. Reboot.