**************************************************************

*    Installation for NetWare DOS ODI Workstation Driver     *

**************************************************************

The workstation driver file is stored in the \DOS_ODI subdirectory.

1. Power on the workstation, and execute Novell's LSL program.

2. At the DOS prompt type the following command to execute the NetWare

workstation driver.

[drive]:\DOS_ODI\E935

where [drive] is the floppy disk drive that this diskete is in.

3. After executing the driver, run NetWare's IPXODI.COM & NETX.EXE

(or VLM.EXE) files.

4. Please refer to Novell's Installation Manual for more details.

*******************************************************

*     Driver Installation for Personal NetWare 1.x    *

*******************************************************

The driver for Personal NetWare 1.x is stored in the \DOS_ODI subdirectory.

Installation Procedure:

1. First install Personal NetWare.

2. Copy the driver file to the Personal NetWare subdirectory.

At the DOS prompt, type the following commands:

COPY A:\DOS_ODI\E935.INS C:\NWCLIENT

COPY A:\DOS_ODI\E935.COM C:\NWCLIENT

NOTE: This assumes the diskette is in Drive A:.

3. Update new version LSL.COM to the Personal NetWare subdirectory.

COPY A:\DOS_ODI\LSL.COM C:\NWCLIENT

4. (a).Run the Personal NetWare SETUP.EXE to enter the main menu.

(b).Select "Primary Network Interface Card" item, and press ENTER to

select "CNet CN935 PCI Bus Ethernet Adapter".

(c).Return to the main menu to select "Configure Primary Interface Card"

to configure the "Media Frame Type(s)".

(d).At the main menu, press "Save Changes and Exit".

5. Execute STARTNET.BAT.