******************************************

*    Installation for Windows NT 4.0	 *

******************************************

1. Insert the driver floppy disk into drive A: or B:.

2. Double click the "Control Panel" in the "My Computer" Window after

Windows NT has been started.

3. Choose the "Network" icon in the "Control Panel" window, and click

"Adapters".

4. Click "Add..." button, and then click "Have Disk..." button.

5. Input the drive A: or B: with path, \WINNT\40.  The network driver will

then be copied to the hard disk.

6. After users see the "CNet CN935 PCI Ethernet Adapter" message, click

the OK button.

7. In the CN935 PCI Adapter Setup window, the cable selection

will appear on screen.  There are AutoSense, Twisted-Pair

(10Base T), Twisted-Pair Full_Duplex, Twisted-Pair No_Link_Test,

and BNC (10Base2). Choose one and click OK botton.

The files will automatically be copied to the hard disk.

8. Return to the Network window and then click OK button.

9. Reboot.