*****************************************************

*                                                   *

*  CNet CN40BC Netware Device Driver Installation   *

*                                                   *

*****************************************************

Netware ODI driver for OS/2 Installation Guide

==============================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files from the

"CNet Setup & Driver Diskette" to the "C:\COPEN" directory of your

hard disk. To copy the file, type:

COPY A:\COPEN.OS2 C:\COPEN

A. Netware Driver for OS/2 Installation

7)   First, install NetWare Requester for OS/2.

8)   Copy E200P driver to the subdirectory of NetWare.

At the OS/2 prompt, type the following commands:

COPY A:\OS2_ODI\E200P.SYS C:\NETWARE\

9)   Use a text editor to modify C:\CONFIG.SYS as follows:

...

...

>> DEVICE=C:\COPEN\COPEN.OS2 IO=xxx /IRQ=yy /O  <<--- add this line

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

>> DEVICE=C:\NETWARE\NE2000.SYS << ---  modify NE2000.SYS to E200P.SYS

...

...

where the value xxx must be the same as the PORT value, and

yy must match the INT value in step 10).

NOTE: There is a text editor named E.EXE to use under OS/2 full screen.

Example:

At the OS/2 prompt, type:

E C:\CONFIG.SYS

10)  Use a text editor to create C:\NETWARE\NET.CFG.

The following is an example of the NET.CFG file:

Link driver E200P

INT 5

PORT 300

FRAME Ethernet_802.3

NOTE: There is a text editor named E.EXE to use under OS/2 full screen.

Example:

At the OS/2 prompt, type:

E C:\NETWARE\NET.CFG

11)  Shut down and reboot the computer.