Windows for Workgroups 3.11 Driver

==================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the Setup & Driver Diskette to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. Windows for Workgroups

9)   To install drivers in Windows for Workgroups, you should be in the

Windows environment first and choose the Network Setup icon.

10)  Click on "Drivers" in the Network Setup window.

11)  Choose the "Add Adapter" item in the Drivers window.

12)  When the new menu is displayed, choose the "Unlisted or Updated Network

Adapter" item for the new driver.

13)  There is a box for entering the device and directory. Please key in

A:\WFW311  <enter>

The next step inputs the IRQ channel and the I/O address.

If users select the NDIS driver, the IO Base and IRQ values could be

zeros for autosense. Users can also set the pertinent values for the

settings by using the SETUP program.

14)  In the "Driver Type" box, the options are "Real Mode and Enchanced Mode

NDIS Driver", "Enchanced Mode NDIS Driver", "Real Mode NDIS Driver" or

"ODI Driver".

a) If you choose "ODI Driver", copy A:\DOS_ODI\E200P.COM

and A:\DOS_ODI\NET.CFG files to your hard disk and append the

E200P.COM statement in the AUTOEXEC.BAT file.

b) If any of the three NDIS Driver options are selected, in the

PROTOCOL.INI file, the IOBASE value MUST be the same as that of

the "/IO=" option (xxx) in the COPEN.EXE command line from step 8).

The INTERRUPT value MUST be the same as that of the "/IRQ="

option (yy) in the COPEN.EXE command line from step 8).

Also, if NDIS is in real mode, be sure that the COPEN.EXE command

precedes the E200P.DOS command in the C:\CONFIG.SYS file.

15)  Now, exit Windows for Workgroups and reboot.