***************************************************

*						      *

*	CN40BC NDIS 3.1 Device Driver Installation    *

*						      *

***************************************************

Installation of E200P.SYS onto Windows NT 4.0

===============================================

The CN40BC PCMCIA Ethernet Adapter can work with Windows NT 4.0.

Please follow these instructions to load this driver into your Windows NT

system:

1.	Logon as Administrator and select the "Network" icon from the

"Control Panel" in "Main" window.

2.	a. On the "network" dialogue box, select "adapter", then click

"add" button.

b. On the "Select Network Adapter" dialogue box, click "Have

Disk" button.

c. On the "Insert Disk" dialogue box, type Drive:\WINNT\40,

then click "OK" button, where Drive is A or B.

d. After "CNet CN40BC PCMCIA Ethernet Adapter" shows up, click "OK".

e. On the "PCMCIA Network Card Setup" dialogue box, select

a set of I/O and IRQ number which does not conflict with

other devices. Normally, you click "OK" on this dialogue box.

(Note: For version A and B cards, please only select I/O with 300

and do NOT select other I/O ports. For version C cards, you

can select I/O ports between 200-3FF. On the back side of the

card, you can see the version differences:

A. Version A card with "PCMCIA Card Type II" wording,

B. Version B card with "PCMCIA Card (B) Type II" wording,

C. Version C card with "PCMCIA Card (C) Type II" wording).

f. On the "PCMCIA Ethernet Adapter Bus Location" dialogue box,

select "PCMCIA" bus on the type item (click downward arrow key).

(NOTE: If you do not see the "PCMCIA" bus on the "Type" item,

cancel this "network" adapter addtion procedure. This

is due to that PCMCIA bus is not booted up by NT. To

solve this, go to "Control Panel", then click "Devices"

icon. In this dialogue box, select "Pcmcia" device. then

click "Startup" button. On the "Device" dialogue box,

select "System", then click OK. Shotdown NT and reboot it

to bring up PCMCIA bus. After PC starts up, select

"Network" icon in the "Control Panel" again. On the "Network"

dialogue, select "CNet CN40BC PCMCIA Ethernet Adapter" item,

click "Properties" to start "PCMCIA" bus type again.)

3. After the adapter addition is completed, still, on the "Network"

dialogue box, click "bindings" button. This action allows to bind

the driver and protocols you are going to choose.

4. Still on the "Network" dialogue box, select "protocols" to make sure

you have all protocols in the box. If you want to access Novell

NetWare, select "NWLink IPX/SPX COmpatible Transport", then

click "Properties". On the NWLink IPX/SPX Properties" dialogue

box, select a correct frame type (802.3 for NetWare 3.11

type (802.3 for NetWare 3.11 as default, 802.2 for NetWare 3.12

and above as default), then click OK.

5. Click "CLose" button on the "Network" dialogue box, then restart NT

to start the network.