***************************************************

*						      *

*	CN40BC NDIS 3.0 Device Driver Installation    *

*						      *

***************************************************

Installation of E200P.SYS onto Windows NT 3.51

===============================================

The CN40BC PCMCIA Ethernet Adapter can work with Windows NT 3.51,

1995 final release.

Please follow these instructions to load this driver into your Windows NT

system:

1.	Logon as Administrator and select the "Network" icon from the

"Control Panel" in "Main" window.

2.	Select the "Add Adapter..." button in "Network Settings" subwindow,

highlight the "<Other> Requires disk from manufacturer" item in

"Add Network Adapter" subwindow, and click "Continue" button.

3.	Insert the diskette with the appropriate driver in drive A or B.

4.	Follow the instructions appearing on screen, "Insert Disk" subwindow,

and select the appropriate drive name (A:\ or B:\) when required to

give the location for the driver and the OEMSETUP.INF file.

For example :

A:\WINNT\351 <--- Directory of CNet CN40BC PCMCIA Ethernet Driver

5.	After pressing the "Continue" button, confirm the driver diskette

with the message "CNet CN40BC PCMCIA Ethernet Adapter" in the "Select

OEM Option" subwindow, set the "IRQ Level" and "I/O Port Address",

(IRQ=5, I/O=300 as default) and click the "OK"  button in the

"CNet CN40BC Network Card Setup" subwindow.

(Note: For version A and B cards, please only select I/O with 300 and

do NOT select other I/O ports. For version C cards, you can

select I/O ports between 200-3FF. On the back side of the

card, you can see the version differences:

A. Version A card with "PCMCIA Card Type II" wording,

B. Version B card with "PCMCIA Card (B) Type II" wording,

C. Version C card with "PCMCIA Card (C) Type II" wording).

6.	In the "Bus Type" subwindow, set the "Bus Location" to "PCMCIA",

and click "OK" button.

7.	Now, the driver has been installed.  Please check the "CNet CN40BC

PCMCIA Ethernet Adapter" message in the "Installed Adapter Card" box

of the "Network Settings" window.  If users want to select the

pertinent protocol drivers, please select "Bindings..." button.

After done, click the "OK" button from the "Network Settings" window.

The protocol and MAC drivers will bind together.

8.	Reboot