NetWare DOS ODI Workstation Driver

**********************************

INSTALLATION PROCEDURE

1)   Insert CNet's CN40BC PC Card Ethernet adapter into your

computer's Type II PC card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (usually C:\NETWARE)

to hold the Novell NetWare utilities. Then change to that

directory.

To make a directory, type:

MD C:\NETWARE

To change to that directory, type:

CD C:\NETWARE

6)   Copy the Novell Netware and PC Card adapter's enabling files

from this Setup & Driver Diskette to the "C:\NETWARE" directory of

your hard disk. The files are:

SETUP.EXE

COPEN.EXE

LSL.COM

E200P.COM

IPXODI.COM

NETX.EXE

NET.CFG

STARTNET.BAT

To copy these files, type:

COPY A:\SETUP.EXE C:\NETWARE

COPY A:\COPEN.EXE C:\NETWARE

COPY A:\DOS_ODI\LSL.COM C:\NETWARE

COPY A:\DOS_ODI\E200P.COM C:\NETWARE

COPY A:\DOS_ODI\IPXODI.COM C:\NETWARE

COPY A:\DOS_ODI\NETX.COM C:\NETWARE

COPY A:\DOS_ODI\NET.CFG C:\NETWARE

COPY A:\DOS_ODI\STARTNET.BAT C:\NETWARE

(The versions of LSL.COM, IPXODI.COM and NETX.EXE included in your

Novell NetWare may be used in place of the files on this diskette.)

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\NETWARE\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots the PowerNIC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can run

LAN drivers.)

9)   Edit the NET.CFG file using a text editor.

a) Change the "PORT" value in the file to that of the "/IO=" option

(xxx) in the COPEN.EXE command line from step 8.

b) Also, change the "INT" value of the file if necessary to the

same as that of the "IRQ=" option (yy) in the COPEN.EXE command

line.

c) Lastly, check with your network administrator to make sure

that the "FRAME" value in the file is Ethernet_802.3. The default

value is "Ethernet_802.3". However, if you are running NetWare

version 3.12 and above, you may have to change this line to:

FRAME Ethernet_802.2

10)  The following commands must be executed to connect to the

NetWare server:

LSL

E200P

IPXODI

NETX

The file STARTNET.BAT includes these commands for your convenience.

At the DOS prompt, simply type STARTNET <enter> to access the NetWare

server.

If you are running NetWare version 3.12 and above, you should

change the last line from "NETX" to "VLM". (Note: You may have to

install the VLM files by applying the "NetWare Client for DOS"

diskettes.)