NetWare v4.x or v3.12 Server Driver

***********************************

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can run

LAN drivers.)

B. NetWare v4.x or v3.12

9)   Power on the file server, and execute the Novell SERVER.EXE program.

10)  At the NetWare system prompt, type

LOAD A:\NW_4\E200P.LAN

a.  When the I/O address is requested during loading, the input

value MUST be the same as that of "/IO=" option (xxx)

in the COPEN.EXE command line from step 8).

b.  When the Interrupt line is requested during loading, the input

value MUST be the same as that of "/IRQ=" option (yy)

in the COPEN.EXE command line from step 8).

11)  The server program will load and execute the E200P.LAN driver module.

Please refer to the NetWare v4.x or v3.12 Installation Manual for more

information on making an auto-execution batch file to load the

E200P.LAN module.