****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

PathWorks for DOS Driver

========================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. NDIS Driver Installation

9)   Run the "NETSETUP.EXE" program in the "Client Setup" diskette.

10)  Choose the default value "Non-LK250 keyboard" as the keyboard type.

11)  Select "DECNET" in the "Network Transport" screen.

12)  Insert the "PWRKS V4.1 DNET TRNSPRT" diskette into the disk drive.

13)  Assign a destination drive (default is C:).

14)  The following are the values to input for the installation dialog.

<1> Create an initial workstation diskette: "NO"

<2> Input workstation node name and address.

<3> Use asynchronous DECNET (ASYNC): "NO"

<4> Use only DECNET DOS components: "NO"

<5> Input server node name and address.

<6> Append LK250 keyboard setup info to the

AUTOEXEC.BAT file: "NO"

<7> Select network adapter type: "ETHERNET"

<8> Choose the "Other NDIS" item and input the NDIS driver path to be

A:\ or B:\ plus "NDIS_20\E200P.DOS"

<9> Input "A:\NDIS_20\" to get the path of the PROTOCOL.INI file

<10> Add the system service DOS subdirectory to

the DOS search path: "NO"

15)  In the CONFIG.SYS file, modify the statement

"SHELL=C:\COMMAND.COM C:\DOS\ /P"  to be

"SHELL=C:\COMMAND.COM C:\DOS\ /P /E:319" .

The value of the "/E" option should not be less than 319.

16)  The PROTOCOL.INI file may need to be revised. Make sure the

IRQ, NI_IRQ variable, is in agreement with the PC Card's IRQ.

The IOBASE value MUST be the same as that of "/IO=" option (xxx)

in the COPEN.EXE command line from Step 8) above.

The INTERRUPT value MUST be the same as that of "/IRQ=" option (yy)

in the same command line.

17)  Make sure the sequence of command lines in the C:\CONFIG.SYS

file is similar to the example below:

:           :         :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:           :         :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\DECNET\E200P.DOS

:           :         :