****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

>> THIS FILE CONTAINS INSTRUCTIONS FOR BOTH THE OS/2 AND DOS ENVIRONMENTS. <<

>> THE DOS PROCEDURE FOLLOWS THE OS/2 SECTION.                             <<

******************************************************************************

PC/TCP for OS/2 v1.2 pl 1

==========================

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Copy A:\COPEN.OS2 to C:\PCTCP\DRV

(where "A:" is the drive this diskette is in)

6)   Copy A:\NDIS_20\E200P.OS2 to C:\PCTCP\DRV

(where "A:" is the drive this diskette is in)

7)   Add the following lines to the CONFIG.SYS file:

DEVICE=C:\PCTCP\DRV\COPEN.OS2 [/IO=xxx] [IRQ=yy] /O

DEVICE=C:\PCTCP\DRV\E200P.OS2

For example,

DEVICE=C:\PCTCP\DRV\COPEN.OS2 [/IO=xxx] [IRQ=yy] /O

DEVICE=C:\PCTCP\DRV\PROTMAN.OS2 /I:C:\PCTCP\DRV

DEVICE=C:\PCTCP\DRV\E200P.OS2

DEVICE=C:\PCTCP\DRV\SOCKET.OS2

Where:

xxx is the I/O base address (300, 320, 340 or 360)

yy is Interrupt line (3, 4, 5, 9, 10, 12, or 15)

Default values are 300 and 5 if /IO=xxx or IRQ=yy are not typed.

8)   Edit the C:\PCTCP\DRV\PROTOCOL.INI file:

For example,

;************************

;**     ADAPTERS       **

;************************

;  CN40BC using 200EP drivers

;

[E200P_NIF]

DRIVERNAME = E200P$

INTERRUPT = 5              ; This value must match the yy value above

IOBASE = 0x300             ; This value must match the xxx value above

;************************

;**     PROTOCOLS      **

;************************

;FTP PC/TCP for OS/2

[SOCKET]

drivername = $SOCKET

bindings = E200P_NIF

*******************************************************************************

PC/TCP v2.2 for DOS Driver

==========================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. NDIS DOS Driver Installation

9)   Create a directory named LANMAN on the hard disk which consists of

relevant files from the PCTCP diskette labeled "PC Network Software

Unsupported Network Software Diskette A".

10)  Run INSTALL.EXE in the PCTCP directory.

11)  Choose the "NDIS Driver" item from the "Network Interface Card

Driver Selection" window. Press F10.

12)  Input the driver pathname "\NDIS_20\E200P.DOS" for this diskette

in the "Interface Driver" window.

13)  Key in the complete path and file name,"C:\LANMAN\PROTMAN.DOS".

14)  Enter the complete path and file name, "C:\LANMAN\NETBIND.EXE".

15)  Modify the content of the PROTOCOL.INI file to:

[E200P] ------------------------->  [E200P_NIF]

16)  In the PROTOCOL.INI file, the IOBASE value MUST be the same as

that of the "/IO=" option (xxx) in the COPEN.EXE command line

from Step 8) above.

The INTERRUPT value MUST be the same as that of the "/IRQ="

option (yy) in the same command line.

17)  Append the content of the CONFIG.INS file in the PCTCP directory to

C:\CONFIG.SYS. Add one more option in the following statement.

DEVICE=C:\LANMAN\PROTMAN.DOS  ------------------> Old Version

DEVICE=C:\LANMAN\PROTMAN.DOS /I:C:\LANMAN  -----> New Version

18)  Make sure the sequence of command lines in the C:\CONFIG.SYS

file is similar to the example below:

:           :         :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:           :         :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\LANMAN\E200P.DOS

:           :         :

19)  Copy the contents of the PCTCP\AEXECFTP.BAT file to AUTOEXEC.BAT