Readme File for NDIS Driver Usage

=======================================

PAGE 1

1. Introduction:

The NDIS lan driver is designed for E-LAN-2000 series adapter.

The device driver conforms to the Microsoft/3Com Network Driver

Interface Specification(NDIS) which is a de facto standard in

the industry. The driver can be used in the DOS workstation of

LAN Manager or any network O.S. which support NDIS protocol.

2. Diskette Directory Structure:

The following directory structure is identical to the installation

diskettes of the LAN Manager with two exceptions. One is an added

level at the top of the directory named MSLANMAN.DOS, and the other

is an added readme file README.DOC in MSLANMAN.DOS directory.

<ROOT>

�

���MSLANMAN.DOS

�

����DRIVERS

�    �

�    ����ETHERNET

�    �	�

�    �	����EL2000

�    �	       �

�    �	       �����EL2000.DOS

�    �	       �

�    �	       �����PROTOCOL.INI

�    �

�    �����NIF

� 	 �

� 	 ����EL2000.NIF

�

����README.DOC

The following describes the directory contents under the DRIVERS tree:

NIF	 Contains the NDIS Network Information Files that are

read by the setup utility.

ETHERNET Contains NDIS drivers and Protocol.ini files for

Ethernet cards. Each NDIS driver should have it's own

subdirectory by the name of the associated NIF file.

PAGE 2

3. NIF File Formats:

There are two information files in this diskette: EL2000.NIF and

PROTOCOL.INI. The EL2000.NIF file provides the information for

setup utility needs to coordinate and install EL2000.DOS driver. The

PROTOCOL.INI file is used by the LANMAN Protocol manager and netbind

program at startup time. This file contains the NDIS and Transport

specific parameters for the network.

>> NIF Format:

Model = E-LAN-2000 adapter

Path = ethernet\EL2000

DeviceDriver = EL2000.dos

Type = NDIS

The following is a description of each field.

Model	: A descriptive remark about the card. This entry is free form

and may contain any characters.

Path	: Location of the files that are pertinent to the driver, i.e,

the Protocol.ini file and the driver itself.

DeviceDriver: The name of the device driver that is added to config.sys.

Type	: Specifies the type of driver. (always assign NDIS)

>> PROTOCOL.INI:

A protocol.ini file must be created for the NDIS driver. The format for

the file is shown below.

IOBase = 0x300

Interrupt = 3

DriverName = EL2000$

This file describes the hardware configuration for your E-LAN-2000

Ethernet adapter, and the DriverName should assign a character string

(less than 8 octects) as a name for device driver EL2000.DOS.

PAGE 3

4. Installing the new driver:

a. After installation of your DOS Workstation of LAN Manager

, change the directory to the default root pathname

LANMAN.DOS and run utility SETUP.

b. Press enter to skip Full Help message box and select "Actions"

item:

�������������������������Microsoft LAN Manager SETUP���������������������������

�Actions�   Options    Exit

���������������������������Ŀ

�  Install...		    �

�  View/modify...	    �

�  Save 		    �

���������������������������ĳ

�  Remove		    �

�  Detach		    �

�  Attach		    �

���������������������������ĳ

�  Import network drivers   �

���������������������������ĳ

�  About Setup...	    �

�����������������������������

c. Select "Import network drivers" subitem and press enter to skip Full

Help message box and a message box will display like below.

��������������������������Microsoft LAN Manager SETUP�������������������F1=Help

Actions    Options	Exit

�� Import Network Drivers Ŀ

�			     �

� Source Drive: [A:]	     �

��������������������������ĳ

� #  OK  #  <Cancel>	     �

����������������������������

PAGE 4

d. You can decide for which drive you want to import network

drivers. When the drive has been selected a message box will show:

��������������������������Microsoft LAN Manager SETUP�������������������F1=Help

Actions    Options	Exit

���������������������������Ŀ

�	    Insert	     �

�   a network driver disk   �

�	 in drive A:	     �

�     and press ENTER	     �

���������������������������ĳ

� #  OK  #  <Cancel>	     �

�����������������������������

e. You must follow the indication of above message to insert

driver disk in drive a: and press ENTER then SETUP utility will

read the information file in driver disk and show the following

box.

��������������������������Microsoft LAN Manager SETUP�������������������F1=Help

Actions    Options	Exit

�������������� Import Network Drivers ������������Ŀ

� 						 �

� Network adapter cards:				 �

�  ��������������������������������������������Ŀ  �

�  �[ ] E-LAN-2000 adapter		      �  �

�  �					      �  �

�  �					      �  �

�  �					      �  �

�  ����������������������������������������������  �

� 						 �

� Protocols:					 �

�  ������������������������������������������Ŀ	 �

�  �#................(Empty)................  �	 �

�  �					    �	 �

�  �					    �	 �

�  �					    �	 �

�  ��������������������������������������������	 �

��������������������������������������������������ĳ

� #  OK  #  <Cancel>				 �

����������������������������������������������������

PAGE 5

f. Press Space Bar to select the driver (there are only one choice in

above message box) and press enter to download the device driver

from diskette to hard disk.

g. After above steps have been taken, you can return to the main menu

(refer b.) and select the subitem "View/modify..." to install this

driver on your LAN Manager DOS Workstation (refer LAN Manager

Installation Guide & Network Device Driver Guide in detail).