******************************************************************************

*	      Genius LAN GE2000III Ethernet Card WINDOWS 95 Driver	      *

******************************************************************************

Introduction

============

This document describes the procedures to install Windows 95 driver

for GE2000III Series Ethernet Adapter.

Windows 95 is the operation system with Plug&Play feature.	If you set

the adapter in Plug&Play mode, Windows 95 will auto-detect your card's

configuration and set it.  The user don't need to do anything for the

adapter configuration or installing the driver. If you set the network

card in Jumperless mode, you must setup driver as following procedures

by manual.

Installation Procedures

=======================

1. When log into Windows 95, click on the "Start" icon ,then move the

mouse to "Settings" icon and click on the "Control Panel".

2. In the Control Panel window, click on the "Network" icon.

3. In the Network Configuration dialog box, choose the "Add.." button.

4. When the Select Network Component Type dialog box appears, select

"Adapter" icon and then press "Add" button.

5. In the Selected Network adapters dialog box, Choose the "Have disk"

button.

6. Insert the driver diskette into drive A or B.

7. When the Install From Disk dialog box appear, specify the driver path

(A:\WIN95 or B:\WIN95) and then choose "OK" button.

8. Then, you will see "GE2000III Series Ethernet Adapter" list in the

dialog box, choose "OK" button.

9. The Windows 95 OS begin to copy driver from diskette and some system

files from Windows 95 installation CD or diskette.

10. Then, you will see "GE2000III Series Ethernet Adapter" list in the

Network Configuration dialog box.

11. Restarting your system you will acquire network service.