******************************************************************************

*          Genius LAN GE2000III Ethernet Card -- NDIS OS/2 Driver            *

******************************************************************************

A. Diskette Introduction :

==========================

This sub-directory contains the following structures:

MSLANMAN.OS2

|

|

|---DRIVERS

|

|--NIF         : Contains the Network Information File (NIF)

|                that setup needs.

|--ETHERNET

|

|--GE3ND  : Contains the GE2000III NDIS OS/2 driver and

protocol.ini file that could be used for setup.

B. Installation Guildline:

==========================

1. In the OS/2 WINDOW, change to LANMAN subdirectory and run SETUP

program, like as below:

C:\LANMAN>SETUP

2. Press Alt+C to select the "Configuration".

3. Press "D" or move the cursor to "Network Driver" item and press

ENTER.

4. Press Alt+A to select <Add New Config> .

5. In the "Network Adapter Drivers" menu, press Alt+D to select other

driver.

6. Insert the GE2000III driver diskette to the floppy disk A, and press

ENTER.

7. In "Other Network Adapter Drivers" menu, select "Genius LAN GE2000III

Card" and press ENTER.

8. Select the network protocol that you want and press ENTER.

9. TO verify the workstation configuration is correct and press ENTER.

10. Select "SAVE" to save the Lan Manager configuration. It will update

the AUTOEXEC.BAT and CONFIG.SYS files.

11. Exit Lan Manager setup program and reboot the system.

======================================================================