*****************************************************************************

*	    Genius LAN GE2000III Ethernet Card -- LANtastic driver	    *

*****************************************************************************

A. Install Artisoft's LANtastic 4.x & 5.x driver

================================================

To install the GE2000III Plug&Play Ethernet Card under the Artisoft's

LANtastic 4.x and 5.x please follow the instructions given below:

1. Insert the GE2000III driver disk into the system's floppy drive.

2. On the network card selection menu, select any of the available network

card on the available list.

3. Follow LANTASTIC's software setup prodedure to complete software

installation.

4. When LANTASTIC software is completed, then copy the GE2000III LANtastic

driver files to the subdirectory where the software located (default

is C:\LANTASI ). In addtion, there are two different driver files for

different versions of LANTASTIC software (4.x & 5.x) under two sub-

directory of LANTASTIC directory in the GE2000III Plug&Play LAN Card's

driver diskette.

for example, the LANTASTIC software is V4.0, then copy GE2000III driver

files under 4.x subdirectory in its driver diskette:

>COPY A:\LANTASTI\4.x\*.* C:\LANTASTI

5. Return to the DOS prompt and use a text editor to modify the

STARTNET.BAT file. The following is an example of a STARTNET.BAT

file.

@ECHO OFF

PATH C:\LANTASTI;%PATH%

PNPLAN			   <-------- Modify this line as "PNPLAN"

AILANDIO/MPX=C7

REDIR <MACHINE NAME> LOGINS=2

NET LPT TIMEOUT 10

6. If the driver is installed successfully, the PNPLAN.EXE driver will

produce the message below when STARTNET.BAT is executed:

" GE2000III Card AI-LANBIOS Compatible Driver installed "

B. If you want to use NDIS driver to install the driver for GE2000III

on LANtastic 6.x, please refer to the installation documentation in

the 6.X subdirectory on the driver diskette.