3/5/1996

SCO UNIX Ethernet Driver

GE2000III SCO Ehternet Adapter Installation

==================================================

The following files will be used in this installation:

Files          Contents

-----------    -----------------------------------------

driver.o       GE2000III SCO Ethernet Adapter Driver Program

setup          Transfer dos format to unix format setup program

info           Used by netconfig

init           Used by netconfig

master         Used by netconfig

node           Used by netconfig

reconf         Used by netconfig

remove         Used by netconfig

space.c        Used by netconfig

system         Used by netconfig

dummy.o        No netconfig

Mkdev          No netconfig

install        No netconfig

install.ID     No netconfig

install.sco    No netconfig

make           No netconfig

macstat.h      No netconfig

---------------------------------------------------------

1. Login as root user. (On maintenance mode)

2. (a) Installtion Diskette in UNIX format.

# custom

Select Install

Select A New Product

Select entire to install all of the packages

(b) Installation Diskette in DOS format.

Put GE2000III SCO UNIX Driver diskette to floppy A: and use doscp

command to copy GE2000III SCO UNIX Driver into UNIX's directories.

# cd /

# doscp A:/sco/setup /setup

# chmod +x /setup

# ./setup

4. (a) Use netconfig utility (after SCO UNIX Release 3.2 V 4.2)

Avaiable options:

# netconfig

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

4) Quit                                          Select 1. Add a chain

(b) No netconfig utility (before SCO UNIX Release 3.2 V 4.2)

# cd /pnp

# ./install

5. Select top level of chain to Add or q to quit:       4. sco_tcp

(SCO TCP/IP for UNIX)

6. Select next level of chain to Add or q to quit:      Select GE2000III Ehternet

Adapter Driver board 0

7. Add chain sco_tcp -> pnp0 (y/n)                      type 'y'

8. Enter the internet Address of this interface         n.n.n.n

9. Enter the netmask for this interface                 (default 255.255.255.0)

10. Does the interface use a broadcast address of all 1's (y/n)  default : y

11. Enter the boardcast address for this interface       default: n.n.n.n

12. Are these values correct ? (y/n)                    y

13. Pseudo ttys are currently configured, do you want to:

1) Add Pseudo ttys

2) Remove Pseudo ttys

Select an option or enter q to quit [q]          q

14.Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure a element in a chain

q. Quit                                          Select option: q

15. Do you want to relink to kernel now ?               y

16. Do you want this kernel to boot by default (y/n)    y

17. Do you want the kernel environment rebuilt (y/n)    y

18. Reboot unix

# shutdown -y -g0