This version of pcntpk has been modified to have a GPSI keyword on

the command line, and to have two additional receive modes.

set_rcv_mode(7) will give you all individually addressed frames plus

all broadcast frames plus any runt frames (same as mode 3 plus runts).

set_rcv_mode(8) will give you all frames plus any runt frames (same

as mode 6 plus runts).

The standard pktmode that's part of the 11.x packet driver collection

will let you set the driver into receive mode 7 or 8.

When you transmit a packet in either run accept mode, the packet size

is not rounded up to 60.

-russ