ECHO TEST

This is a system level test for the adapter. The workstation sends

packets to the echo server and the echo server echoes the packets

back to the workstation. It is a system integrity test which can find

the hardware status between workstation and workstation, or between

workstation and server.

Follow these instructions to run the Echo Test.

1. Select one networked computer in which has been installed CARDWARE

Ethernet card(CWL9504, CWL9510, or CWL9406 etc) as Echo Server.

2. Insert the User Configuration/Diagnostics diskette to the

floppy drive A: or drive B:.

3. Type 'Install', then select "F6 - Echo Test" from Main Menu.

4. Select "F2 - Set up Echo Server" from Echo Test window, then

a "Echo Test (Server Station)" window will show on the screen.

5. Select another networked computer in which has been installed

CWL9504 PCMCIA Ethernet card as Echo Client

6. Repeat the step 2 and the step 3.

7. Select "F1- Run Echo Test" from Echo Test window. An "Echo Test

(Client Station)" window will show on the screen.

8. At this point, you will see the number of Transmitted/Received

packets shown on the Client Station and Server Station windows.

If the hardware between the Client and the Server is good, the

Packets number is increasing and the suggested Packets number

is 1000. If the Packets number is hung up at 0 or any number, it

means that there is a bad Cable or a unstable Hub exists on the

path. Try to simplify and isolate the path.

Ex: Just setup the Echo Test Path with a server station,

a client station, and a Hub. Or, connect a client station

to a server station by a coaxial cable with two terminators.

9. Any Station can be setup as Client station or Server station.

If the Server station is setup, the rest of client stations can

send/receive packets through that server sequentially.