���������������������������������������������������ķ

�    Release Notes for CWL9504 Ethernet Adapter     �

�                 Cardware Lab Inc.                 �

���������������������������������������������������ͼ

This file provides up-to-date information after the user's manual was

printed. It also provides the information which is not mentioned in the

user's manual.

1. User's Manual correction:

2. Read the LICENSE.DOC file for the license agreement.

3. Read the THISDISK.DOC file to check out the contents of the CardWare

diskette.

4. To install and test CWL9504 adapter, execute CWL9500.EXE under the INSTALL

subdirectory of this diskette. A batch file INSTALL.BAT in the root

directory is provided to invoke CWL9500.EXE for your convenience.

5. NDIS 4.0 miniport driver for Windows NT 4.0/3.51 and Windows 95 is in

the \WINNT and \WIN95 subdirectory respectively. Read the README.DOC

file in that subdirectory for installing the driver.

6. CardWare Lab provides ODI driver and NDIS drivers for the associated

Network Operating System.

7. Technical Support:

BBS   : 408-774-9103

(set your modem to 8 data bits, no parity, and 1 stop bit)

Email : tech@cwli.com

Fax   : 408-774-9102

URL   : http://www.cwli.com/

Free Hot Line: 1-888-CWL-TECH

8. The Patch file for Cirrus Logic PCMCIA Controller

SYMPTOM:

The PCMCIA can be recognized and enabled by CWL9504.EXE

or third party PCMCIA Enabler. However, the Network Driver

can not be initialized due to RAM Failure, or once logged on,

network connection is unreliable.

This is due to a timing problem with a particular revision of the

Cirrus Logic PCMCIA Controller inside certain Notebook PCs.

Notebooks might be affected by this problem are:

PANASONIC CF-41, TI 4000M, NEC 2000, ACER 760, etc.

To check what kind of PCMCIA Controller is built in your system,

either ask the manufacturer or check SS* line in the CONFIG.SYS

file. The SS* line in CONFIG.SYS will be similar to the following.

DEVICEHIGH=C:\CARDWIZ\SSCIRRUS.EXE /SKT:2

SOLUTION:

CIRRPTCH.COM can be used to correct the timing problem

existing in a certain Cirrus Logic PCMCIA Controller.

1.  Copy A:\PATCH\CIRRPTCH.COM onto drive C:\

2.  Add CIRRPTCH.COM to your AUTOEXEC.BAT

3.  Reboot your PC

NOTE: The CIRRPTCH.COM will automatically be installed in the

AUTOEXEC.BAT file under DOS environment. In all other

network environments, you have to manually install it.

This has to be done before the network driver or network

batch file is executed.

9. SPECIAL DRIVERS FOR TOSHIBA T3300SL

The Toshiba T3300SL was one of the first notebooks on the market

to contain a Version 2 PCMCIA socket capable of working with I/O

cards.

As a result of its early introduction, the Toshiba T3300SL does not

fully comply with the PCMCIA specification as it stands today.

To get around this problem, we provide special ODI, NDIS and

Packet drivers for it. These can be used with all major networks.

For details of installing these drivers, either refer to your manual,

or check on-line help.

Remember to replace the following files.

FROM:                      TO:

A:\T3300\CWLODI.COM        A:\NETWARE\ODI\CWLODI.COM

A:\T3300\CWLPD.COM         A:\FTP_TCP\CWLPKT.COM

A:\T3300\CWLNDIS.DOS       A:\NDIS\WFW\CWLNDIS.DOS

A:\T3300\OEMSETUP.INF      A:\OEMSETUP.INF

A:\T3300\CWLNDIS.DOS       A:\NDIS\DOS\CWLNDIS.DOS

A:\T3300\PROTOCOL.INI      A:\NDIS\DOS\PROTOCOL.INI

Note: All other Toshiba PCMCIA Portables introduced since

Feb. 1, 1993 have fully compatible PCMCIA specification.

DO NOT use the drivers in this directory for these machines.