This file contains instructions which tell you how to install

Windows NT LAN driver for CWL9504 LAN PC Card.

Follow these instructions to install the LAN driver:

Windows NT 4.0

==============

1. Select "Network" icon from the Control Panel.

2. From the Network Settings window, select "Adapters" button,

then click "Add...".

3. Click "Have Disk..." button from the Select Network Adapter

dialog box, and then click "Continue" button.

4. Insert the CWL9504 User diskette to the floppy drive.

5. In the Insert Disk dialog box, type A:\WINNT or B:\WINNT and

click "OK" button.

6. Select "CardWareLab CWL9504 Ethernet PC Card" from the Select

OEM Option window, then click "OK" button.

7. In the Input Adapter ID window, you are being asked to specify

the node number of the card. Do one of the followings:

- Select the "Skip" button if there is only one card on the

station.

- Specify the node number of the card if there are several cards

on the system.

(If you do not know the node number, run Diagnostics from the

CWL9504 User diskette to find out.)

8. To check the appropriate hardware settings for the CWL9504 PCMCIA

Ethernet Card, select "Configure" button from the Network Setting

window. Be sure "CardWareLab CWL9504 Ethernet PC Card" shows on

the Installed Adapter Cards window.

9. Select OK button from the Network setttings window. A dialog box

appears and prompts you to restart your computer.

10. Choose "Restart Computer" buttton to reboot the station.

Windows NT 3.51

===============

1. Select "Network" icon from the Control Panel.

2. Click "Add Adapter..." button from the Network Settings window.

3. Select "<Other> Requires disk from manufacturer" from the Add

Network Adapter dialog box, and then click "Continue" button.

4-10. The same procedures of Windows NT 4.0.