***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NetWare OS/2 ODI Workstation             *

***********************************************

The NetWare ODI driver for OS/2 is stored in the OS2_ODI subdirectory

of the CN930 diskette.

1. First, install NetWare Requester for OS/2 in the PC.

2. Copy E930 driver to the subdirectory of NetWare.

At the OS/2 prompt, type the following commands:

COPY A:\OS2_ODI\CN930.SYS C:\NETWARE\

NOTE: This assumes the diskette is in drive A.

3. Use a text editor to modify C:\CONFIG.SYS as follows:

...

...

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

>> DEVICE=C:\NETWARE\NE2000.SYS << ---  change NE2000.SYS to E930.SYS

...

...

4. Use a text editor to create the file NET.CFG in the C:\NETWARE subdirectory.

The following is an example of the C:\NETWARE\NET.CFG file:

Link driver E930

FRAME Ethernet_802.3

5. Shut down and reboot the computer.

NOTE: There is a text editor named E.EXE to use under OS/2 full screen.

To edit the two files above, at the OS/2 prompt, type:

E C:\CONFIG.SYS

or

E C:\NETWARE\NET.CFG