***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NDIS 2.0.1 DOS Driver		      *

***********************************************

The DOS NDIS driver for PC/TCP is located in the \NDIS_20 subdirectory

of the CN930 diskette.

1. Create the subdirectory named LANMAN in the hard disk which consists of

relevant files from the NDIS directory of "PC Network Software

Unsupported Network Software, Diskette A".

2. Run INSTALL.EXE in PCTCP directory.

3. From the "Network Interface Card Driver Selection" window, choose

the item "NDIS Driver". Then, press F10.

4. In the "Interface Driver" window, enter the driver pathname

"A:\NDIS_20\E930.DOS" (assuming this diskette is in drive A).

5. Key in complete pathname with file of "C:\LANMAN\PROTMAN.DOS".

6. Enter complete pathname with file of "C:\LANMAN\NETBIND.EXE".

7. Modify the contents of PROTOCOL.INI:

Change  [E930]  To  [E930_NIF]

8. Append the contents of CONFIG.INS file in the PCTCP directory to

C:\CONFIG.SYS. Add one more option in the following statement.

Change:

DEVICE=C:\LANMAN\PROTMAN.DOS

To:

DEVICE=C:\LANMAN\PROTMAN.DOS /I:C:\LANMAN

9. Copy the PCTCP\AEXECFTP.BAT file to AUTOEXEC.BAT.

***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NDIS 2.0.1 OS/2 Driver		      *

***********************************************

The OS/2 NDIS driver for PC/TCP is located in the \NDIS_20 subdirectory

of the CN930 diskette.

1. Copy A:\NDIS_20\E930.OS2 to C:\PCTCP\DRV

(where "A:" is the drive this diskette is in)

2. Add the following line to the CONFIG.SYS file --

DEVICE=C:\PCTCP\DRV\E930.OS2

Example:

DEVICE=C:\PCTCP\DRV\PROTMAN.OS2 /I:C:\PCTCP\DRV

DEVICE=C:\PCTCP\DRV\E930.OS2

DEVICE=C:\PCTCP\DRV\SOCKET.OS2

3. Edit the C:\PCTCP\DRV\PROTOCOL.INI file to add the correct adapter

and protocol information.

Example:

;========================

;++	  ADAPTERS	++

;========================

;  CN930 PCI Ethernet Adapter

;

[E930_NIF]

DRIVERNAME = E930$

AdapterID = @0080AD123456  ; This item is only required when more

; than one adapter exist on the PC.

;========================

;++	 PROTOCOLS	++

;========================

;FTP PC/TCP for OS/2

[SOCKET]

drivername = $SOCKET

bindings = E930_NIF