***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > LAN Server 			      *

***********************************************

This document describes the procedure to install NDIS driver on DOS

LAN Requester and OS/2 LAN server 2.0/3.0 and 4.0 for CN930 Series

PCI Ethernet Adapter.

**********************

* LAN Server 2.0/3.0 *

**********************

Driver installation procedure on DOS LAN Requester :

----------------------------------------------------

1. Insert the IBM LAN Support Program (LSP) Version 1.30 (or 1.31)

Diskette into your floppy drive A: .

2. Type A:\dxmaid  and press <Enter> .

3. The screen will display the IBM logo. Press <Enter> twice until

the "Environment Information" appears on screen.

4. Responds to the screen prompts, specifying the following values

for each field,use the <Space Bar> to toggle between choices:

Configuration for this computer?	       Yes

Use existing configuration information?   Yes

Do you have adapter option diskettes?     Yes

Are you configuring for two adapters?     No

Do you need 802.2 interface support?      Yes

Source for LSP			      A:\

Target for new configuration	      C:\LSP

Make sure that the values given above are correct, then press

<Enter> to store your choices.

5. Insert the network driver diskette in floppy A, and specify

your pathname A:\NDIS_20, then press ENTER.

6. Follow screen instructions to complete the process. Simply

pressing <Enter> for each screen display will select the default

options. Press <Enter> until you reach the display prompting you

to insert the LSP diskette.

7. Insert the LSP diskette as prompted. Press <Enter> to acknowledge

completion of the action.

8. A screen showing the current configuration detected will be

displayed. It should show the network adapter and the

protocols already selected (i.e., "IBM IEEE 802.2" and

"IBM OS/2 NETBIOS").

9.  Press <F4>. to save the new configuration. This completes the LSP

installation.

10. Save the configuration and reboot your computer.

11. Insert the DOS LAN Requester Install Diskette 1 into your floppy

drive.

12. Type A:\INSTALL	  <Enter>

13. The IBM logo will again appear on the screen. Press <Enter> to

move on to the next screen.

14. Follow screen instructions, pressing <Enter> to move from

screen-to-screen until you are prompted to indicate the pathname

of the DOS LAN Requester directory (C:\DOSLAN ).

15. Specify the pathname (C:\DOSLAN is the default) for the DOS LAN

Requester directory. Then press <Enter> to continue.

16. The program will ask you for the services that you require.

17. Highlight the appropriate option. Select either "Send messages"

or "Send, View, Edit, and Log message" option. (Normally, second

option "Send, View, Edit and Log message" is selected).

18. Specify the PC's ID and domain ID.

19. Follow screen prompts to complete the installation.

Driver installation procedure on OS/2 LAN Server 2.0/3.0 :

----------------------------------------------------------

Before you start with the installation process, make sure that

OS/2 LAN Server is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. Copy E930.OS2 and E930.NIF files from your Driver diskette

(\NDIS_20\IBMLANSE) to C:\IBMCOM\MACS.

2. Click the OS/2 LAN Service icon.

3. Select "Installation and configuration". The IBM logo should

appear onscreen.

4. Click <OK>.

5. Select <Advance> and press <Enter>.

6. Select "Install or Configuration this Workstation."

7. Select server type, adding a server or domain controller.  The

"install or remove" selections should appear.

8. Select "Configure a component."

10. Click the "Configure" button.

11. Select "Configure workstation."

12. Select "CNet CN930 Series PCI Ethernet Adapter" when prompted to

identify  you Network Adapter. Then click the <Add> button.

13. Select IBM NETBIOS from the list and click the <Add> button.

14. Select <OK> to confirm all selections.

15. Follow screen instructions to complete the process, selecting

<OK> to accept default selections.

16. Continue pressing <OK> until your screen prompts you to backup

old  CONFIG.SYS and  AUTOEXEC.BAT and create a new  CONFIG.SYS

and AUTOEXEC.BAT. Also select <OK> to confirm this.

17. Installation is complete now. Reset your PC by turning it off

and on.

Note:

-----

The DOS version must be bellow DOS 5.0 when install DOS LAN requester

into your system.

-------------------------------------------------------------------------------

******************

* LAN Server 4.0 *

******************

Driver installation procedure on DOS LAN Requester :

----------------------------------------------------

Before you start with the installation process, make sure that

DOS LAN Services is properly installed.  Similarly, your adapter

should also be properly installed in your client PC.

1. Copy E930.DOS from your Driver diskette (A:\NDIS_20)

to DOS LAN Services subdirectory C:\DLS .

2. Under DOS LAN Requester subdirectory C:\DLS, run INSTALL

3. The screen display some description. If it's correct, move

highlight to "The listed options are correct" item ,then press

ENTER.

4. Move highlight to "Network Card", then press ENTER.

5. Select "Change driver for network card." item, then press ENTER.

6. Select "Network card not shown in list below..", then press ENTER.

7. Insert the network driver diskette in floppy A,and specify

your pathname A:\NDIS_20\IBMLANSE, then press ENTER.

8. CN930 Ethernet driver is now installed on your computer, reboot

your computer.

NOTES:

------

Parameters for the PROTOCOL.INI file:

AdapterID : This keyword is provided by E930, and is required

when more than one adapter exist on one system.

Driver installation procedure on OS/2 LAN Server 4.0 :

------------------------------------------------------

Before you start with the installation process, make sure that

OS/2 LAN Server is properly installed. Similarly, your adapter

should also be properly installed in your server.

1. In OS/2 window, Click the "MPTS" icon.

2. The screen will display the IBM Log, press "OK".

3. Select "Install" button.

4. Insert the network driver diskette, specify the driver

path A:\NDIS_20 and press <Enter>.

5. When complete above step to load E930.OS2 into OS/2 system,

then configure your system.

6. The same step as step 3, select "Configure" button.

7. The screen will display "Configure" dialog box and select "LAN

adapters and protocols" then press "configure" button.

8. The screen will display "LAPS Configuration" dialog box, move

highlight to "CNet CN930 Series PCI Ethernet Adapter" item in "Network

Adapter" window, and select "NetBios" protocol. Press "OK".

9. Installation is complete now. Reset your PC by turning it off

and on.