***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > SCO UNIX 4.X			      *

***********************************************

The SCO UNIX file is stored in the \SCOUNIX\4.X subdirectory of the CN930

diskette.

Before you start with the installation process, make sure that the

SCO UNIX system is properly installed. Similarly, your adapter should

also be properly installed in your machine.

1. Login as root user. (On maintenance mode)

2. If you own installation Diskette in DOS format :

Put CN930 Network Driver diskette to floppy A: and use doscp

command to copy the file "setup" for installing SCO UNIX Network

PCI Driver into UNIX's directories as following :

# cd /

# doscp A:/scounix/4.x/setup /setup

# chmod +x /setup

# ./setup

3. Use netconfig utility, available options :

# netconfig

The screen will display the following messages, and Select

item (1) to Add a chain	:

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

4) Quit

4. After the above option completed, the screen display next option

messages :

Select item ( ) : sco_tcp   SCO TCP/IP for UNIX

5. When the user completed  the  above option, then various adapter

drivers in SCO UNIX occur and select " CNet CN930 Series PCI Ethernet

Driver board 0" item.

6. Add chain sco_tcp -> r1e0 (y/n),   TYPE 'y'

7. Enter the internet Address of this interface ,

for example : 192.72.53.55

8. Enter the netmask for this interface (default 255.255.255.0).

9. Does the interface use a broadcast address of all 1's (y/n),

default : y

10. Enter the boardcast address for this interface,

default : xxx.xx.x.255	       press ENTER

11. Are these values correct ? (y/n)   TYPE 'y'

12. Pseudo ttys are currently configured, do you want to:

1) Add Pseudo ttys

2) Remove Pseudo ttys

Select an option or enter q to quit [q]	 TYPE 'q'

TCP connections currently configured, do you want to:

1) Add TCP Connections

2) Remove TCP Connections

Select an option or enter q to quit [q]	 TYPE 'q'

13. Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure a element in a chain

q. Quit

Select option: q

14. Do you want to relink to kernel now ?		y

15. Do you want this kernel to boot by default (y/n)	y

16. Do you want the kernel environment rebuilt (y/n)	y

17. Reboot unix.

# shutdown -y -g0