***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > Banyan VINES 4.10			      *

***********************************************

The driver for Banyan VINES is located in the \NDIS_20 subdirectory

of the CN930 diskette.

1. Copy the E930.DOS driver from this diskette (\NDIS_20\E930.DOS) to the

VINES directory on the hard disk which contains the file BAN.EXE.

2. Create a LANMAN subdirectory under the root, and copy the PROTOCOL.INI file

from the \NDIS_20 subdirectory of the CN930 diskette to this directory.

3. Add the following two lines to your CONFIG.SYS file:

DEVICE=<VINES directory>\PROTMAN.DOS

DEVICE=<VINES directory>\E930.DOS

4. Check the PROTOCOL.INI file to make sure the value in the AdapterID

field if multi-CN930 LAN cards exist.

5. Run PCCONFIG and configure the NDIS Ethernet driver.