***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > Wollongong Pathway 		      *

***********************************************

The driver for Wollongong Pathway is located in the \NDIS_20 subdirectory

of the CN930 diskette.

1. Make a directory called NDIS on your hard disk.

2. Copy E930.DOS and PROTOCOL.INI from the \NDIS_20 subdirectory of this

CN930 diskette to the C:\NDIS directory.

3. Change diskettes and run the Pathway Setup Program PWSETUP.

4. After the "Pathway Setup Program for DOS" window appears, choose

"Installation".

5. In "Installation", select "Pathway Run Time". From "Pathway Run Time",

choose "Quick Setup for NDIS".

6. In the "Pathway Runtime Quick Installation for NDIS" window

input the appropriate floppy disk drive letter as the source drive.

The destination should be "C:\PATHWAY".

Also, set the IP address and the number of subnet mask bits.

7. In the "Driver NDIS and Kernel Configuation" window, enter the path

"C:\NDIS" to the device driver and the PROTOCOL.INI file.

8. There is a screen editor for modifying the configuations with respect to

DriverName and AdapterID in PROTOCOL.INI file. Press the F4 key to exit

the editor. AdapterID is only required when more than one adapter exist

on the PC.

Example:

[E930]			   ; NOTE: Add the [E930] section name.

DriverName = E930$

AdapterID = @0080AD123456  ; This item is only required when more than

; one adapter exist on the same system.

9. Next, use the select bar to change network frame type.

10. The Pathway Setup program will copy all relevant files and modify

the CONFIG.SYS file as well as the AUTOEXEC.BAT file automatically.

11. Reboot.